# 3D Printing Summary

| Metrics | Enclosure |
| --- | --- |
| Total Print Time (hr:min) | 4:42 |
| Total Number of Components | 2 |
| Typical Total Mass (g) | 62.6 |
| Typical Number of Print Setups | 1 |

# 3D Printing Settings

| Print File Name | Qty | Total Print Time (hr:min) | Mass (g) | Infill (%) | Support (Y/N) | Layer Height/ Nozzle Diameter(mm) | Notes (orientation, special settings, etc.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Enclosure_Top v0.2.stl | 1 | 2:31 | 35.36 | 20 | N | 0.2/0.4 | - Print in given orientation |
| Enclosure_Bottom v0.2.stl | 1 | 2:11 | 27.24 | 20 | N | 0.2/0.4 | - Print in given orientation |
| Optional Prints |  |  |  |  |  |  |  |
| Joystick_Camera_Mount_ Adapter v0.2.stl | 1 | 0:54 | 8.5 | 20 | Y | 0.2/0.4 | - Print in given orientation |
| Convex_Topper v0.2.stl | 1 | 0:29 | 6.46 | 20 | N | 0.2/0.4 | - Print in given orientation |
| Concave_Topper v0.2.stl | 1 | 0:42 | 8.46 | 20 | N | 0.2/0.4 | - Print in given orientation |
| Cylindrical_Topper v0.2.stl | 1 | 2:18 | 22.29 | 20 | N | 0.2/0.4 | - Print in given orientation - Print with 6 perimeters |
| Goalpost_Topper v0.2.stl | 1 | 3:25 | 35.84 | 20 | Y | 0.2/0.4 | - Print in given orientation - Put a support blocker on the central mounting hole |

# Post-Processing

- Remove any bumps or zits from the surface where the enclosure meets the lid.

- Remove any supports from the camera mount and goalpost topper if printed.

# Customization Options

- Joystick housing can be printed in multiple colours.

- The joystick camera mount adapter is an optional add-on for mounting using ¼-20 threads

- The optional topper prints can be used to customize the joystick to users. Extra materials such as foam, felt, instamorph, etc. can also be added to further customize the joystick toppers.

# Examples of Quality Prints

Photos of the Enclosure

Photos of Toppers

Photos of Camera Mount